A SystemC™ OCP Transaction Level Communication Channel

V1.1 – July 18, 2003

Norman Weyrich, Synopsys

Anssi Haverinen, Nokia

Alan Kamas, Sonics

Yann Bajot, Prosilog

Stephane Guntz, Prosilog

Joe Chou, Sonics

Revision History

Version	Date	Comment

1.0	1/15/03	Initial Generic Transaction Channel

1.0.1	3/31/03	First revision for OCP 1.0 channel

1.1	7/18/03	OCP 1.0 sideband and layer adapters included

OCP International Partnership (OCP-IP) disclaims all warranties and liability for the use of this document and the information contained herein and assumes no responsibility for any errors that may appear in this document, nor does OCP-IP make a commitment to update the information contained herein.

Contact the OCP-IP office to obtain the latest revision of this document.

Questions regarding this document or membership in OCP-IP may be forwarded to:

OCP-IP

www.ocpip.org

E-mail: admin@ocpip.org

Phone: +1 503-291-2560

Fax: +1 503-297-1090

OCP-IP Technical Support

techsupport@ocpip.org

DISCLAIMER

This OCP-IP document is provided "as is" with no warranties whatsoever, including any warranty of merchantability, noninfringement, fitness for any particular purpose, or any warranty otherwise arising out of any proposal, specification or sample.  OCP-IP disclaims all liability for infringement of proprietary rights, relating to use of information in this document.  No license, express or implied, by estoppel or otherwise, to any intellectual property rights is granted herein.

All product names are trademarks, registered trademarks, or servicemarks of their respective owners.

Copyright (C) 2003 OCP-IP

# Introduction

This document describes the API of SystemC channel implementing an OCP-based transaction level communication interface. This is an extension to “SystemC™ Generic Transaction Level Communication Channel” (www.systemc.org, Contributions area).

Throughout the paper, we use the categorization of communication abstraction levels introduced in white paper “SystemC™ based SoC Communication Modeling for the OCP™ Protocol” (www.ocpip.org):

- Transaction Level

  - Layer-3: Message Layer

Model untimed functionality

Point-point communication

  - Layer-2: Transaction Layer

Model/analyze SoC architecture
Start SW development

Estimate timing

  - Layer-1: Transfer Layer

Cycle true but faster than RTL

Detailed analysis, develop low-level SW

- Pin Level

  - Layer-0: Register Transfer Level

“TLx” and Layer-x are used for Transaction Level, Layer-x interchangeably. The model files referred here are stored in package “ocp_tl_sc_v1.1.tar.gz”.

# Transaction CHannel

The transaction channel (in header file tl_channel.h) is a new version of the generic OSCI transaction channel, with minor additions explained in the next section. The channel provides synchronization for transactions, and it is used with all transaction layers. The channel can support both clocked and event-driven master and slave modules. The channel is templatized over a protocol-specific data class, which in this case contains the member functions for implementing OCP protocol (in header files ocp_tl1_data_cl.h and ocp_tl2_data_cl.h).

The channel synchronization methods and events are visible to the masters and slaves through SystemC interface definitions (in files tl_master_if.h, tl_slave_if.h and tl_direct_if.h).

The implementation of the channel is somewhat different from the original generic channel, because of better support for clocked TL1 masters and slaves, and for providing easier integration with RTL models. The channel implementation is not meant to be user-modifiable, and should not matter to the users. For those interested, more information can be found in the channel heading and in-line comments.

# SystemC API of the channel

In this section, we provide the API of the generic communication channel.  In particular, we describe the constructor parameters of the channel and the methods of the channel’s master and slave interface. This is followed by a description of the methods of the data classes for the OCP protocol.

## Constructor Parameters of the Channel

The channel has the following constructor:

TL_Channel(sc_module_name name,

bool Synchron = false,

bool SyncEvent = true,

bool DefaultEvent = true)

- name: Name of the module (channel) instance.

- Synchron specifies whether the channel’s internal states and events are updated synchronously (Synchron = true) or asynchronously (Synchron = false). The latter is slightly faster. In most applications, the update mechanism makes no difference. Synchron = true should always be used with OCP data classes.

- SyncEvent specifies whether the channel’s events for the synchronization of Mput*() and Sget*() as well as Sput*() and Mget*() methods are triggered (SyncEvents = true) or not (SyncEvents = false). The Channel is faster if no synchronization events are used. This could be used e.g. in a TL1 application where only non-blocking methods are employed. If blocking methods and/or *releasePE() calls are used SyncEvent must be true. Use the default value (= true) unless you know exactly what you are doing.

- DefaultEvent specifies whether the channel should trigger the default event. The channel is faster if no default event is triggered. DefaultEvent can be false if none of the attached modules is sensitive to port event. Use the default value (= true) unless you know exactly what you are doing.

## Channel API Methods

The channel is templatized over the data class. The data class itself is templatized over the data type and the address type. In the following TdataCl denotes the template data class argument (template<class TdataCl> class TL_Channel).

Even though the API calls are many, the use of these is extremely simple, and only a couple of calls are needed for each master and slave. See simple examples for reference (directory tl_sc/simple_examples).

### Master Interface (tl_master_if.h)

TdataCl * GetDataCl()

Purpose:	Gets the pointer to the data class of the channel.

Return:	Returns the pointer immediately.

bool MgetSbusy ()

Purpose:	Status of the slave-busy semaphore. Indicates whether the slave has released the previous request.

Return:	Immediately returns true if slave has not responded to the last request event, and false if it has.

Events:	No event.

bool MgetSbusyData ()		//New for this release

Purpose:	Status of the data-busy semaphore. Indicates whether the slave has released the previous data request. Used only with TL1 protocols, which have separate data and request phases.

Return:	Immediately returns true if slave has not responded to the last data request event, and false if it has.

Events:	No event.

bool MputWriteRequestBlocking ()

Purpose:	Issues a write request to the slave. The master may write to the channel’s data class any time outside this call by either copying, or pointer passing. MgetSbusy() always returns false when this call is used.

Return:	Suspends calling thread. Resumes and returns after the slave has released the request channel. Returns true at success, false at failure.

Events:	Triggers the default event, which can be sensed by the slave (and master).

bool MputWriteRequest()

Purpose:	Issues a write request to the slave. Should be called only after MgetSbusy()  returns false, i.e. the slave is no longer using the channel data buffer. Both copy and pointer-passing put data methods can be used, but the passed data buffer must not be re-used by the master until MgetSbusy () is false.

Return:	Returns immediately. Master must suspend itself to allow slave process to run. Returns true if channel accepts the request, false if not.

Events:	Causes the default event to slave (and master). No event when return vaue is false.

bool MputDataRequestBlocking ()		//New for this release

Purpose:	Issues a data write request to slave. The master may write to the data handshake group of the channel’s data class any time outside this call by either copying, or pointer passing. MgetSbusyData() always returns false when this call is used. Used only with TL1 protocols, which have separate data and request phases.

Return:	Suspends calling thread. Returns after the slave has released the data request channel. Returns true at success, false at failure.

Events:	Triggers the default event, which can be sensed by the slave (and master).

bool MputDataRequest()		//New for this release

Purpose:	Issues a data write request to slave. Should be called only after MgetSbusy()  returns false, i.e. the slave is no longer using the channel data buffer. Both copy and pointer-passing put data methods can be used, but the passed data buffer must not be re-used by the master until MgetSbusyData () is false. Used only with TL1 protocols, which have separate data and request phases.

Return:	Returns immediately. Master must suspend itself to allow slave process to run. Returns true if channel accepts the request, false if not.

Events:	Causes the default event to slave (and master). No event when return-value is false.

bool MputReadRequestBlocking ()

Purpose:	Issues a read request to slave. Master may read the response data only after MgetResponse*() call returns true. MgetSbusy() always returns false when this call is used.

Return:	Suspends calling thread. Returns after the slave has released the request channel. Returns true at success and false at failure.

Events:	Causes the default event to slave (and master).

bool MputReadRequest ()

Purpose:	Issues a read request to the slave. Should be called only when MgetSbusy() returns false. The master may read the response data only after a MgetResponse*() call returns true.

Return:	Returns immediately. Master must suspend itself to allow slave process to run. Returns true if channel accepts the request, false if not.

Events:	Causes the default event to slave (and master). No event if return value is false.

bool MgetResponseBlocking (bool Release)

Purpose:	Gets response from the slave. Can only be called after Mput*Request(). The channel’s data class contains valid data, and the channel’s data pointer can be used by the master any time outside this call, before Mput*Request() is called again.

Parameters:	If Release is true, the response channel is released immediately. Otherwise the response channel is not released until Mrelease() is called.

Return:	Suspends calling thread. Returns after the slave has called the SputResponse() method. True at success, false at failure.

Events:	No event.

.

bool MgetResponse (bool Release)

Purpose:	Gets response from slave. Can only be called after Mput*Request(). The channel’s data class contains valid data, and the channel’s data pointer can be used by the master when this call returns true, and before Mput*Request() is called again.

Parameters:	If Release is true, the response channel is released immediately. Otherwise the response channel is not released until Mrelease() is called.

Return:	Returns immediately. Returns true after the slave has responded to the read request, false before. Master must suspend itself to allow slave process to run.

Events:	No event.

bool MgetResponsePE ()

Purpose:	Gets response from slave. Can only be called after Mput*Request(). The channel’s data class contains valid data, and the channel’s data pointer can be used by the master when this call returns true, and before Mput*Request() is called again. Use only in conjunction with MreleasePE(). Use only with clocked processes.

Parameters:	None.

Return:	Returns immediately. Returns true after the slave has responded to the read request, false before. Master must suspend itself to allow slave process to run.

Events:	No event.

void Mrelease ()

Purpose:	Releases the response channel. Notice that calling thread is not suspended, and slave thread cannot run until master suspends itself.

Return:	Returns immediately. No return value.

Events:	No event.

void MreleasePE ()		//New for this release

Purpose:	Release response channel pre-emptively, that is without knowing if there is a response. This is used in fully synchronous TL1 masters, which do not want to block the response. The primary purpose of this method is to allow fully synchronous TL1 masters use single-cycle response handshake (no channel events are needed). This causes SgetMbusy() return true right after the SputResponse*() call. Causes MgetRequest() to return false, so the existence of active response must be tested with MgetRequestPE(). See examples for further guidance.

Return:	Returns immediately. No return value.

Events:	No event.

void Mrelease (sc_time Time)

Purpose:	Releases the response after Time time units. Notice that calling thread is not suspended, and slave thread cannot run until master suspends itself. This call is primarily used with TL2 and TL3.

Return:	Returns immediately. No return value.

Events:	No event.

void  MregisterDirectIF (MdirectIF<TdataCl > *MasterDirectIF)

Purpose:	This method registers the direct interface of the master at the channel. It must be called if the master has implemented the SputDirect() method, which can be used by the slave to directly read or write data to the master without affecting the timing of the system.

Return:	Returns immediately. No return value.

Events:	No event.

bool MputDirect(int MasterID, bool IsWrite, Td* DataPointer, Ta Address, int NumWords)

Purpose:	This method belongs to the direct interface of the slave. It must be implemented in the slave. It allows the master to directly read or write data to the slave without affecting the timing of the system. If the slave has implemented this method, the slave must register the method at the channel by calling the SregisterDirectIF() method of the channel.

Return:	Returns immediately. True at success, false at failure. A return of false usually means that the slave has not implemented this method, i.e. the slave does not support direct access.

Events:	No event.

### Slave Interface (tl_slave_if.h)

TdataCl * GetDataCl()

Purpose:	Gets the pointer to the data class of the channel.

Return:	Returns the pointer immediately.

bool IsWrite()

Purpose:	Indicates whether current request is a read or a write transfer. Other request types can be communicated over the data class (in case of OCP, through MCmd field).

Return:	false = Read transfer

true =  Write transfer

bool SgetMbusy ()

Purpose:	Status of the master busy semaphore. Indicates whether the master has released the previous request.

Return:	Immediately returns true if master has not received the last response event, and false if it has.

Events:	No event.

bool SgetRequestBlocking(bool Release)

Purpose:	Block execution until master signals a request event. Channel data can be used, until slave thread suspends (meets a wait() call).

Parameters:	If Release is true, the request channel is released immediately. Otherwise the request channel is not released until Srelease() is called.

Return:	Suspends calling thread. Returns after read or write event from master.

Events:	No events.

bool SgetRequest(bool Release)

Purpose:	Get request. If true, channel data can be used, until slave thread suspends (i.e. meets a wait() call).

Parameters:	If Release is set true, the request channel is released immediately. Otherwise it is not released until Srelease() is called.

Return:	Returns true when master request is pending, returns immediately.

Events:	No events.

bool SgetRequestPE()			//New for this release

Purpose:	Get request. If true, channel data can be used, until slave thread suspends (i.e. meets a wait() call). Use only with SreleasePE(). Use only with clocked processes.

Parameters:	None.

Return:	Returns true when master request is pending, returns immediately.

Events:	No events.

bool SgetDataRequest(bool Release) 		//New for this release

Purpose:	Get data request. If true, channel data can be used, until slave thread suspends (i.e. meets a wait() call).

Parameters:	If Release is set true, the data request channel is released immediately. Otherwise it is not released until SreleaseData() is called. Used only with TL1 protocols, which have separate data and request phases.

Return:	Returns true when master data request is pending, returns immediately.

Events:	No events.

bool SgetDataRequestPE() 		//New for this release

Purpose:	Get data request. If true, channel data can be used, until slave thread suspends (i.e. meets a wait() call). Use only with SreleaseDataPE(). Use only with clocked processes.

Parameters:	None.

Return:	Returns true when master data request is pending, returns immediately.

Events:	No events.

bool SgetDataRequestBlocking(bool Release) 	//New for this release

Purpose:	Block execution until master signals a data request event. Channel data can be used, until slave thread suspends (meets a wait() call).

Parameters:	If Release is true, the request channel is released immediately. Otherwise the data request channel is not released until SreleaseData() is called. Used only with TL1 protocols, which have separate data and request phases.

Return:	Suspends calling thread. Returns after read or write event from master.

Events:	No events.

bool SputResponseBlocking ()

Purpose:	Issues a response to master. Can be called only after a request is detected by SgetRequest(). The response data can only be written between these get and put calls.

Return:	Suspends calling thread. Returns after the master called Mrelease(). Returns true at success and false at failure.

Events:	Causes a response event to master.

bool SputResponse()

Purpose:	Issues a response to master. Can be called only after a request is detected by SgetRequest() or SgetRequestBlocking(). The response data can only be written between these get and put calls.

Return:	Returns immediately. Returns true if channel accepts the response, false if not.

Events:	Causes a response event to master. No event if return value is false.

void Srelease()

Purpose:	Releases request channel. Notice that calling thread is not suspended, and master thread cannot run until slave suspends itself.

Return:	Returns immediately. No return value.

Events:	No events.

void SreleasePE()		//New for this release

Purpose:	Releases request channel pre-emptively. See MreleasePE().

Return:	Returns immediately. No return value.

Events:	No events.

void Srelease(sc_time Time)

Purpose:	Release request channel after Time time units. Notice that calling thread is not suspended, and master thread cannot run until slave suspends itself.

Return:	Returns immediately. No return value.

Events:	No events.

void SreleaseData()		//New for this release

Purpose:	Releases data request channel. Notice that calling thread is not suspended, and master thread cannot run until slave suspends itself. Used only with TL1 protocols, which have separate data and request phases.

Return:	Returns immediately. No return value.

Events:	No events.

void SreleaseDataPE()		//New for this release

Purpose:	Releases data request channel pre-emptively. Used only with TL1 protocols, which have separate data and request phases. See MreleasePE().

Return:	Returns immediately. No return value.

Events:	No events.

void SreleaseData(sc_time Time) 		//New for this release

Purpose:	Release Datarequest channel after Time time units. Notice that calling thread is not suspended, and master thread cannot run until slave suspends itself. Used only with TL1 protocols, which have separate data and request phases.

Return:	Returns immediately. No return value.

Events:	No events.

void  SregisterDirectIF(SdirectIF<TdataCl > *SlaveDirectIF)

Purpose:	This method registers the direct interface of the slave at the channel. It must be called if the slave has implemented the MputDirect() method, which can be used by the master to directly read or write data to the slave without affecting the timing of the system

Return:	Returns immediately. No return value.

Events:	No event.

bool SputDirect(int SlaveID, bool IsWrite, Td* DataPointer, Ta Address, int NumWords)

Purpose:	This method belongs to the direct interface of the master. It must be implemented in the master. It allows the slave to directly read or write data to the master without affecting the timing of the system. If the master has implemented this method, the master must register the method at the channel by calling the MregisterDirectIF() method of the channel.

Return:	Returns immediately. True at success, false at failure. A return of false usually means, that the master has not implemented this method, i.e. the master does not support direct access.

Events:	No event.

## OCP TL1 Data Class

An OCP TL1 channel can be created by using the generic channel class template with an argument of the OCP_TL1_DataCl class, which is provided in the tl_sc/ocp_tl1_data_cl.h header file. The OCP_TL1_DataCl class contains private member variables and public access methods such that, by using them, OCP dataflow transactions and sideband signals can be exchanged between the master and slave ports.

In general, for each master-driven OCP signal, M*, there is a corresponding MputM*() driving method. In addition, SgetM*() methods also exist for those signals and can be used by the slave to sample the signals. Similarly for slave-driven OCP signals, S*, corresponding SputS*() and MgetS*() methods are provided. As for the OCP control and status sideband signals, access methods for the system side and the core side are distinguished by having the “Sys” prefix and the “C” prefix, respectively.

This naming scheme makes the behavior of most of the channel methods obvious. For those that are not, some explanation is given in the following.

### Enumerator Types

Three enumerator types, OCPMCmdType, OCPSRespType, and OCPMBurstType, are defined in the header file, with the encoding as specified for the OCP MCmd signal, the SResp signal, and the MBurst signal, respectively. The encodings are named according to the OCP specification (www.ocpip.org).

MCmd encoding:

enum OCPMCmdType {

OCP_MCMD_IDLE = 0,

OCP_MCMD_WR,

OCP_MCMD_RD,

OCP_MCMD_RDEX,

OCP_MCMD_RESERVED4,

OCP_MCMD_WRNP,

OCP_MCMD_RESERVED6,

OCP_MCMD_BCST

};

SResp encoding:

enum OCPSRespType {

OCP_SRESP_NULL = 0,

OCP_SRESP_DVA,

OCP_SRESP_RESERVED2,

OCP_SRESP_ERR

};

MBurst encoding:

enum OCPMBurstType {

OCP_MBURST_LAST = 0,

OCP_MBURST_DFLT1,

OCP_MBURST_TWO,

OCP_MBURST_DFLT2,

OCP_MBURST_FOUR,

OCP_MBURST_STRM,

OCP_MBURST_EIGHT,

OCP_MBURST_CONT

};

### Mandatory (and generic) Data Class Member Functions

These seven methods are used by the channel itself, and hence they must always be defined. These methods do not follow the general naming scheme described in the previous sub-section since they are not used through masters or slaves. The private member variable m_MCmd[1 – m_ReqToggle] carries the value shown on the OCP MCmd signal at the beginning of a cycle (or transaction). The three toggle methods, ToggleRequest(), ToggleDataRequest(), and ToggleResponse(), trigger an update process of the data class’ private member variables of the OCP request phase, data phase, and response phase, respectively. The update process can be either an inertial update or an immediate update, depending on the setting of m_Synchron variable. The inertial update ensures that current data members are never updated at the same clock edge they are read. This is required in TL1 for independence of thread execution order.

bool IsWriteRequest()

{

return((m_MCmd[1 – m_ReqToggle] == OCP_MCMD_WR) ? true : false);

}

void SetWriteRequest()

{

m_MCmd[m_ReqToggle] = OCP_MCMD_WR;

}

void SetReadRequest()

{

m_MCmd[m_ReqToggle] = OCP_MCMD_RD;

}

void ToggleRequest()

{

m_Request = true;

if (m_Synchron) request_update();

else update();

}

void ToggleDataRequest()

{

m_DataRequest = true;

if (m_Synchron) request_update();

else update();

}

void ToggleResponse()

{

m_Response = true;

if (m_Synchron) request_update();

else update();

}

### OCP Request Group Signals

The request group signals can be written by the master (Mput*) when the MgetSbusy() channel call returns false, before Mput*Request*(). Typically, the request group signals are written at a rising clock edge or a small delay later (the latter one with RTL/TLM co-simulation).

The signals can be read by the slave (Sget*) when the SgetRequest() channel function returns true, at a clock rising edge, or at channel default event.

void MputMAddr(Ta a)

Ta SgetMAddr()

void MputMAddrSpace(int a)

int SgetMAddrSpace()

void MputMBurst(OCPMBurstType a)

OCPMBurstType SgetMBurst()

void MputMByteEn(int a)

int SgetMByteEn()

void MputMCmd(OCPMCmdType a)

OCPMCmdType SgetMCmd()

Purpose: 	Return the OCP MCmd field. This call resets the MCmd, i.e. the SgetMCmd() is not persistent. This behavior, although different from OCP MCmd signal in RTL level, is all right because the data class calls are not used for synchronization.

void MputMConnID(int a)

int SgetMConnID()

void MputMData(Td d)

void SgetMData(Td &d)

Td SgetMData() // Just a different form of the above

void MputMThreadID(int a)

int SgetMThreadID()

### OCP Data Request Group Signals

The data request group signals can be written by the master (Mput*) when the MgetSbusyData() channel call returns false, before MputDataRequest*(). Typically, the request group signals are written at a rising clock edge or a small delay later (the latter one with RTL/TLM co-simulation). These signals are to be used with OCP interfaces, which have data handshake enabled. Notice that the MputMDataHS() and SgetMDataHS() are used instead of MputMData() and SgetMData(). Those calls should not be intermixed.

The signals can be read by the slave (Sget*) when the SgetDataRequest() channel function returns true, at a clock rising edge, or at channel default event.

void MputMDataHS(Td d)

void SgetMDataHS(Td &d)

Td SgetMDataHS()

void MputMDataValid(bool a)

bool SgetMDataValid()

Purpose: 	Return the OCP MDataValid field. This call resets the MDataValid, i.e. the SgetMDataValid() is not persistent. This behavior, although different from OCP MDataValid signal in RTL level, is all right because the data class calls are not used for synchronization.

void MputMDataThreadID(int a)

int SgetMDataThreadID()

### OCP Response Group Signals

The response group signals can be written by the slave (Sput*) when the SgetMbusy() channel call returns false, before Sput*Response*(). Typically, the request group signals are written at a rising clock edge or a small delay later (the latter one with RTL/TLM co-simulation).

The signals can be read by the master (Mget*) when the MgetResponse() channel function returns true, at a clock rising edge, or at channel default event.

void SputSData(Td d)

void MgetSData(Td &d)

Td MgetSData()

void SputSResp(OCPSRespType a)

OCPSRespType MgetSResp()

Purpose: 	Return the OCP SResp field. This call resets the SResp, i.e. the MgetSResp() is not persistent. This behavior, although different from OCP SResp signal in RTL level, is all right because the data class calls are not used for synchronization.

void SputSThreadID(int a)

int MgetSThreadID()

### Example: Address Transfer Methods

The address transfer methods are shown here as an example to illustrate how the private member variables, m_MAddr[0] and m_MAddr[1], are set when their access methods are called. The MputMAddr() method is used to drive a new address onto the OCP MAddr signal. The SgetMAddr() method is used to sample the OCP MAddr signal. Note that the data class is templatized over the address type (Ta). This allows switching between, e.g., 32-bit addresses and 64-bit addresses without re-writing code. The toggling happens at least one delta cycle after the transaction is initiated through the channel. This provides the necessary inertia, so that data does not just trickle through the channel.

void MputMAddr(Ta a)

{

m_MAddr[m_ReqToggle] = a;

}

Ta SgetMAddr()

{

return m_MAddr[1 – m_ReqToggle];

}

### Sideband Signals

MError

| definition | void MputMError( bool nextValue ) |
| --- | --- |
| called by | Master |
| explanation | Changes the next value of the MError signal. If the OCP channel is asynchronous, then the change is immediate. If the channel is synchronous then the change occurs at the next update. |

| definition | bool SgetMError( ) const |
| --- | --- |
| called by | Slave |
| explanation | Returns the current value of the MError signal in the channel. |

| definition | const sc_event& SidebandMErrorEvent() const |
| --- | --- |
| called by | Slave |
| explanation | Returns the event associated with the MError signal. This event is triggered whenever the MError signal changes to a new value. Note that a call to setMError() or resetMError() will not always result in the SidebandMErrorEvent occurring. For example, if the current value of MError is true and the function setMError() is called, the event SidebandMErrorEvent will not be triggered since the current value (true) and the next value (true) are the same. |

MFlag

| definition | void MputMFlag( int nextValue ) |
| --- | --- |
| called by | Master |
| explanation | Changes the next value of the MFlag signal. If the OCP channel is asynchronous, then the change is immediate. If the channel is synchronous then the change occurs at the next update. |

| definition | int SgetMFlag( ) const |
| --- | --- |
| called by | Slave |
| explanation | Returns the current value of the MFlag signal in the channel. |

| definition | const sc_event& SidebandMFlagEvent() const |
| --- | --- |
| called by | Slave |
| explanation | Returns the event associated with the MFlag signal. This event is triggered whenever the MFlag signal changes to a new value. |

SError

| definition | void SputSError( bool nextValue ) |
| --- | --- |
| called by | Slave |
| explanation | Changes the next value of the SError signal. If the OCP channel is asynchronous, then the change is immediate. If the channel is synchronous then the change occurs at the next update. |

| definition | bool MgetSError( ) const |
| --- | --- |
| called by | Master |
| explanation | Returns the current value of the SError signal in the channel. |

| definition | const sc_event& SidebandSErrorEvent() const |
| --- | --- |
| called by | Master |
| explanation | Returns the event associated with the SError signal. This event is triggered whenever the SError signal changes to a new value. Note that a call to setSError() or resetSError() will not always result in the SidebandSErrorEvent occurring. For example, if the current value of SError is true and the function setSError() is called, the event SidebandSErrorEvent will not be triggered since the current value (true) and the next value (true) are the same. |

SFlag

| definition | void SputSFlag( int nextValue ) |
| --- | --- |
| called by | Slave |
| explanation | Changes the next value of the SFlag signal. If the OCP channel is asynchronous, then the change is immediate. If the channel is synchronous then the change occurs at the next update. |

| definition | int MgetSFlag( ) const |
| --- | --- |
| called by | Master |
| explanation | Returns the current value of the SFlag signal in the channel. |

| definition | const sc_event& SidebandSFlagEvent() const |
| --- | --- |
| called by | Master |
| explanation | Returns the event associated with the SFlag signal. This event is triggered whenever the SFlag signal changes to a new value. |

SInterrupt

| definition | void SputSInterrupt( bool nextValue ) |
| --- | --- |
| called by | Slave |
| explanation | Changes the next value of the SInterrupt signal. If the OCP channel is asynchronous, then the change is immediate. If the channel is synchronous then the change occurs at the next update. |

| definition | bool MgetSInterrupt( ) const |
| --- | --- |
| called by | Master |
| explanation | Returns the current value of the SInterrupt signal in the channel. |

| definition | const sc_event& SidebandSInterruptEvent() const |
| --- | --- |
| called by | Master |
| explanation | Returns the event associated with the SInterrupt signal. This event is triggered whenever the SInterrupt signal changes to a new value. Note that a call to setSInterrupt() or resetSInterrupt() will not always result in the SidebandSInterruptEvent occurring. For example, if the current value of SInterrupt is true and the function setSInterrupt() is called, the event SidebandSInterruptEvent will not be triggered since the current value (true) and the next value (true) are the same. |

Control

| definition | bool SysputControl( int nextValue ) |
| --- | --- |
| called by | System side |
| explanation | If ControlBusy is false, this function changes the next value of the Control sideband signal. If the ControlBusy signal is part of the OCP channel configuration and the current value of ControlBusy is true, then the next value of the Control sideband signal will not be changed and the setControl function will return false. Otherwise, the function will return true and will set the next value of the Control signal. If the OCP channel is asynchronous, the change to the Control signal is immediate. If the channel is synchronous then the change occurs at the next update. |

| definition | int CgetControl( ) const |
| --- | --- |
| called by | Core side |
| explanation | Returns the current value of the Control signal in the channel. |

| definition | const sc_event& SidebandControlEvent() const |
| --- | --- |
| called by | Core side |
| explanation | Returns the event associated with the Control signal. This event is triggered whenever the Control signal changes to a new value. |

ControlWr

| definition | void SysputControlWr( bool nextValue ) |
| --- | --- |
| called by | System side |
| explanation | Changes the next value of the ControlWr signal. If the OCP channel is asynchronous, then the change is immediate. If the channel is synchronous then the change occurs at the next update. |

| definition | bool CgetControlWr( ) const |
| --- | --- |
| called by | Core side |
| explanation | Returns the current value of the ControlWr signal in the channel. |

| definition | const sc_event& SidebandControlWrEvent() const |
| --- | --- |
| called by | Core side |
| explanation | Returns the event associated with the ControlWr signal. This event is triggered whenever the ControlWr signal changes to a new value. [Note: this event might be more useful if it only occurred when ControlWr changes from 0 to 1.] |

ControlBusy

| definition | void CputControlBusy( bool nextValue ) |
| --- | --- |
| called by | Core side |
| explanation | Changes the next value of the ControlBusy signal. If the OCP channel is asynchronous, then the change is immediate. If the channel is synchronous then the change occurs at the next update. |

| definition | bool SysgetControlBusy( ) const |
| --- | --- |
| called by | System side |
| explanation | Returns the current value of the ControlBusy signal in the channel. |

| definition | const sc_event& SidebandControlBusyEvent() const |
| --- | --- |
| called by | System side |
| explanation | Returns the event associated with the ControlBusy signal. This event is triggered whenever the ControlBusy signal changes to a new value. Note that a call to setControlBusy() or resetControlBusy() will not always result in the SidebandControlBusyEvent occurring. For example, if the current value of ControlBusy is true and the function setControlBusy() is called, the event SidebandControlBusyEvent will not be triggered since the current value (true) and the next value (true) are the same. |

Status

| definition | void CputStatus( int nextValue ) |
| --- | --- |
| called by | Core side |
| explanation | This function changes the next value of the Status sideband signal. If the OCP channel is asynchronous, the change to the Status signal is immediate. If the channel is synchronous then the change occurs at the next update. |

| definition | int SysgetStatus( ) const |
| --- | --- |
| called by | System side |
| explanation | Returns the current value of the Status signal in the channel. |

| definition | bool readStatus( int& currentValue ) const |
| --- | --- |
| called by | System side |
| explanation | If the channel signal StatusBusy is false, then this function sets the passed parameter currentValue to the current value of the Status signal in the channel. Then the event SidebandStatusRdEvent is triggered and the function returns true. If the channel signal StatusBusy is true, then the read is not performed, the event SidebandStatusRdEvent is not triggered and the function returns false. |

| definition | const sc_event& SidebandStatusEvent() const |
| --- | --- |
| called by | System side |
| explanation | Returns the event associated with the Status signal. This event is triggered whenever the Control signal changes to a new value. |

StatusRd

| definition | void SysputStatusRd( bool nextValue ) |
| --- | --- |
| called by | System side |
| explanation | Changes the next value of the StatusRd signal. If the OCP channel is asynchronous, then the change is immediate. If the channel is synchronous then the change occurs at the next update. |

| definition | bool CgetStatusRd( ) const |
| --- | --- |
| called by | Core side |
| explanation | Returns the current value of the StatusRd signal in the channel. |

| definition | const sc_event& SidebandStatusRdEvent() const |
| --- | --- |
| called by | Core side |
| explanation | Returns the event associated with the StatusRd signal. This event is triggered whenever the ControlWr signal changes to a new value. [Note: this event might be more useful if it only occurred when StatusRd changes from 0 to 1.] |

StatusBusy

| definition | void CputStatusBusy( bool nextValue ) |
| --- | --- |
| called by | Core side |
| explanation | Changes the next value of the StatusBusy signal. If the OCP channel is asynchronous, then the change is immediate. If the channel is synchronous then the change occurs at the next update. |

| definition | bool SysgetStatusBusy( ) const |
| --- | --- |
| called by | System side |
| explanation | Returns the current value of the StatusBusy signal in the channel. |

| definition | const sc_event& SidebandStatusBusyEvent() const |
| --- | --- |
| called by | System side |
| explanation | Returns the event associated with the StatusBusy signal. This event is triggered whenever the StatusBusy signal changes to a new value. Note that a call to setStatusBusy() or resetStatusBusy() will not always result in the SidebandStausBusyEvent occurring. For example, if the current value of StatusBusy is true and the function setStatusBusy() is called, the event SidebandStatusBusyEvent will not be triggered since the current value (true) and the next value (true) are the same. |

MReset_n

The API for this sideband signal is still to be determined.

SReset_n

The API for this sideband signal is still to be determined.

### TL1 versus RTL

The TL1 protocol sequences are similar to RTL ones, with only small differences. It is possible to re-create accurate OCP timing diagrams with correctly constructed masters and slaves. The OCP fields internal to the data class do not follow OCP timing accurately, because the generic channel synchronization has to be combined with the data class to implement full protocol. For example, the MCmd field is reset when SgetMCmd() is called. Therefore, to re-create the MCmd signal for RTL SgetRequest(), Srelease() and clock must be used.

Since TL1 event model is far simpler than RTL event model for simulation speed purposes, arbitrary RTL delays and signal glitches cannot be fed into TL1 channel. The data fields must remain stable once the transaction is committed. This causes extra difficulties for creating RTL-TLM converters, similar to co-simulation of RTL and cycle-based models. The difficulties rise from the very essense of the abstraction levels, and are unavoidable in our opinion. The RTL side of the RTL-TLM converter must take care of glitch removal.

### Example: Sending and Receiving Write Transactions

The following pseudo-code segments show, as an example, how an OCP write transaction can be sent over an OCP channel using the OCP_TL1_DataCl class’ public methods. It is assumed that this is a posted write transaction, thus, no response sent.

// ----------------------------

// on the master sending side

// ----------------------------

wait(); // Wait for clock rising edge

MputMConnID(0);

MputMThreadID(0);

MputMAddr(0x30);

MputMData((Td)wr_data);

MputWriteRequest();

// ----------------------------

// on the slave receiving side

// ----------------------------

wait(); // Wait for clock rising edge

if (SgetRequest(1)) {

int mconnid = SgetMConnID();

int mthreadid = SgetMThreadID();

Ta address = SgetMAddr();

OCPMCmdType mcmd = SgetMCmd();

Td wr_data = SgetMData();

}

Calling the MputMConnID(), MputMThreadID(), and MputMAddr() methods sets the OCP MConnID, MThreadID, and MAddr signals, respectively. The address passed into the MputMAddr() method should be on an OCP-word boundary. The MputWriteRequest() call indicates that an OCP write transaction is going to be delivered. The MputMData() method is used to send the write data onto the OCP channel.

On the slave side, the SgetMConnID(), SgetMThreadID(), SgetMAddr(), and SgetMCmd() methods are called to retrieve values of the OCP MConnID, MThreadID, MAddr, and MCmd signals, respectively. The SgetMData() method is used to receive the write data.

Note that the slave sees the request at the next clock edge after the master has called MputWriteRequest().

### Example: Sending and Receiving Read Responses

The following pseudo-code segments show, as an example, how a single-OCP-word read transaction can be exchanged between the master and slave of an OCP connection.

// ----------------------------

// on the master sending side

// ----------------------------

// sending a read request

wait(); // Wait for rising clock edge

MputMAddr(0x30);

MputReadRequest();

…..

// receiving a read response and data

if (MgetResponse(1)) {

OCPSRespType sresp = MgetSResp();

If (sresp == OCP_SRESP_DVA) {

Td rd_data = MgetSData();

}

}

// ----------------------------

// on the slave side

// ----------------------------

wait(); // Wait for rising clock edge

if (SgetRequest(1)) {

Ta address = SgetMAddr();

OCPMCmdType mcmd = SgetMCmd();

If (mcmd == OCP_MCMD_RD) {

// Response code (data valid)

SputSResp(OCP_SRESP_DVA);

SputSData(rd_data);

SputResponse();

}

}

This time, the MputMCmd() and MputBurstLen() calls, together, send a single-OCP-word read request to the OCP channel. When the slave receives a request, it checks whether it is a read request first. In this example, after the read data is obtained, the slave sends back an OCP DVA response and the single-OCP-word read data by calling the SputSResp() and SputSData()  methods, respectively.

On the master side, when a read response has arrived, the master uses the MgetSResp() and MgetSData() methods to retrieve the status of the response and the read data, respectively.

## OCP TL2 Data Class

An OCP TL2 channel can be created by using the generic channel class template with an argument of the OCP_TL2_DataCl class, which is provided in the tl_sc/ocp_tl2_data_cl.h header file. The OCP_TL2_DataCl class contains private member variables and public access methods such that, by using them, OCP dataflow transactions and sideband signals can be exchanged between the master and slave ports. Three enumerator types, OCPMCmdType, OCPSRespType, and OCPMBurstType, are also defined in the header file, with the encoding as specified for the OCP MCmd signal, the SResp signal, and the MBurst signal, respectively. See the TL1 data class section.

In general, the TL2 data class looks very similar to the user as the TL1, with a few additional members. Data toggling is not required since task synchronization provided by threads protects the data.

### Required Data Class Member Functions

These are the same as for TL1, but note that the Toggle methods are obsolete with OCP TL2 channel.

bool IsWriteRequest()

{

return((m_MCmd == OCP_MCMD_WR) ? true : false);

}

void SetWriteRequest()

{

m_MCmd = OCP_MCMD_WR;

}

void SetReadRequest()

{

m_MCmd = OCP_MCMD_RD;

}

void ToggleRequest()

{

assert(0);

}

void ToggleDataRequest()

{

assert(0);

}

void ToggleResponse()

{

assert(0);

}

### OCP Request Group Signals

The request group signals can be written by the master (Mput*) right before the Mput*Request*() call. The data protection toggle switches state at each request. The slave may read the signals (Sget*) right after the SgetRequest*() returns true.

The TL2 transaction contains typically a full OCP burst. The data fields are set in the beginning of the burst and stay constant during it. For example, the address field of the transaction is the first address of the burst, and the slave derives the other addresses from the MBurst field. Since some fields, like byte enable, may change at each transfer of a burst, it is possible to break the burst into several transactions when necessary. The shortest transaction contains one OCP transfer (bus cycle). The transaction that contains the last transfer of the burst is marked with setting the last_of_a_burst parameter of the MputData call true.

void MputMAddr(Ta a)

Ta SgetMAddr()

void MputMAddrSpace(int a)

int SgetMAddrSpace()

void MputMBurst(OCPMBurstType a)

OCPMBurstType SgetMBurst()

void m_MputMByteEn(int a)

int SgetMByteEn()

void MputMCmd(OCPMCmdType a)

OCPMCmdType SgetMCmd()

void MputMConnID(int a)

int SgetMConnID()

void MputMData(Td* d, int w = 1, bool last_of_a_burst = true)

Parameters:

d : Pointer to data array

w : Data array length (atomic transaction length)

last_of_a_burst : Last datum of a burst transfer is sent with this transaction

Td* SgetMData(int& w, bool& last_of_a_burst)

Parameters:

w : Data array length (atomic transaction length)

last_of_a_burst : Last datum of a burst transfer is sent with this transaction

Return value: Pointer to data array

Td* SgetMData(int& w)

Parameters:

w : Data array length (atomic transaction length)

Return value: Pointer to data array

void MputMThreadID(int a)

int SgetMThreadID()

void MputAtomicLen(int w)

Purpose:  	Set Request Transaction data length. Can be used in lieu of w-parameter of MputMData() function. Not part of OCP 1.0, but necessary for TL2 to work.

int SgetAtomicLen()

Purpose:   	Return Request Transaction data length. Can be used in lieu of w-parameter of SgetMData() function. Not part of OCP 1.0, but necessary for TL2 to work.

Time stamp methods: These methods can be used by master to indicate duration of request packet. The MputEndTime simulates the time it takes the master to output the current transaction. In other words, the end time is the earliest time the request packet is completely through the interface, if the slave does not do any throttling. The time stamps do not delay channel events; they are meant for additional information for the slave to calculate the release time instant.

void MputEndTime(sc_time tt)

{

ReqEndTime = tt;

}

sc_time SgetEndTime()

{

return ReqEndTime;

}

### OCP Response Group Signals

The response group signals can be written by the slave (Sput*) right before the Sput*Request*() call. The data protection toggle switches state at each response. The master may read the signals (Mget*) right after the MgetRequest*() returns true.

void SputSData(Td* d, int w = 1, bool last_of_a_burst = true)

Parameters:

d : Pointer to data array

w : Data array length (atomic transaction length)

last_of_a_burst : Last datum of a burst transfer is sent with this transaction

Td* MgetSData(int& w, bool& last_of_a_burst)

Parameters:

w : Data array length (atomic transaction length)

last_of_a_burst : Last datum of a burst transfer is sent with this transaction

Return:	Pointer to data array

Td* MgetSData(int& w)

Parameters:

w : Data array length (atomic transaction length)

Return:	Pointer to data array

void SputSResp(OCPSRespType a)

OCPSRespType MgetSResp()

void SputSThreadID(int a)

int MgetSThreadID()

void SputAtomicLen(int w)

Purpose:  	Set Response Transaction data length. Can be used in lieu of w-parameter of SputSData() function.  Not part of OCP 1.0, but necessary for TL2 to work.

int MgetAtomicLen()

Purpose:  	Return Request Transaction data length. Can be used in lieu of w-parameter of MgetSData() function. Not part of OCP 1.0, but necessary for TL2 to work.

Non-OCP fields for running the TL2 example code. These may not be available in future releases:

void SputSAddr(Ta a)

Ta MgetSAddr()

bool IsWriteResponse()

void SputWriteResponse()

void SputReadResponse()

void SputSCmd(OCPMCmdType a)

OCPMCmdType MgetSCmd()

void SputSConnID(int a)

int MgetSConnID()

Time stamp methods: These methods can be used by the slave to indicate duration of response packet. The SputEndTime simulates the time it takes the slave to output the current transaction. In other words, the end time is the earliest time the response packet is completely through the interface, if the master does not do any throttling. The time stamps do not delay channel events; they are meant for additional information for the master to calculate the release time instant.

void SputEndTime(sc_time tt)

{

ResEndTime = tt;

}

sc_time MgetEndTime()

{

return ResEndTime;

}

### Example: Sending and Receiving (Burst) Write Transactions

The following pseudo-code segments show, as an example, how a 16-OCP-word write burst transaction can be sent from (received by) the master (slave) of an OCP channel using the OCP_TL2_DataCl class’ public methods. Assuming this is a posted write transaction, thus, no response is exchanged.

// ----------------------------

// on the master sending side

// ----------------------------

MputMConnID(0);

MputMThreadID(0);

MputMAddr(0x30);

MputMCmd(OCP_MCMD_WR);//Not mandatory since MputWriteRequestBlocking() is used

// assuming wr_data_ptr is pointed to the 16-word write data

MputMData(wr_data_ptr, 16, true);

// the wr_data_ptr and its contest can only be changed after the transaction is committed

…..

MputWriteRequestBlocking();

// ----------------------------

// on the slave receiving side

// ----------------------------

if (SgetRequestBlocking(1)) {

int mconnid = SgetMConnID();

int mthreadid = SgetMThreadID();

Ta address = SgetMAddr();

OCPMCmdType mcmd = SgetMCmd();

Td* wr_data_ptr = SgetMData(burst_word_length,last_chunk_of_a_burst);

// after done with the data pointer, need to commit this write transaction

…..

}

Calling the MputMConnID(), MputMThreadID(), and MputMAddr() methods sets up the OCP MConnID, MThreadID, and MAddr signals, respectively. The address passed into the MputMAddr() method should be on an OCP-word boundary. The MputMCmd() call indicates that an OCP write transaction is going to be delivered. The MputMData() method is used to send a 16-OCP-word chunk of write data of the write burst transaction onto the OCP channel; plus, it is the last chunk of the burst transfer (this is indicated by setting the last actual argument “true”).

On the slave side, the SgetMConnID(), SgetMThreadID(), SgetMAddr(), and SgetMCmd() methods are called to retrieve values of the OCP MConnID, MThreadID, MAddr, and MCmd signals, respectively. The SgetMData() method is used to receive the write data chunk pointer, plus, the data word length, and to tell whether or not this is the last chunk of the current write burst transaction.

Note that for the MputMData() method and the SgetMData method, only pointer passing is allowed in this version. Therefore, the data pointer and its content should not be changed until the transaction is committed (i.e., released). This does not cause any problems since channel uses data toggle for protection. Copying transactions execute considerably slower, and require dynamic memory allocation in the channel.

### Example: Sending and Receiving (Burst) Read Responses

The following pseudo-code segments show, as an example, how a single-OCP-word read transaction can be exchanged between the master and slave of an OCP connection. We also assume that this OCP connection is configured without the MConnID and MThreadID signals.

// ----------------------------

// on the master sending side

// ----------------------------

// sending a read request

MputMAddr(0x30);

MputMCmd(OCP_MCMD_RD);

MputAtomicLen(1);

MputReadRequest();

…..

// receiving a read response and data

if (MgetResponseBlocking(1)) {

OCPSRespType sresp = MgetSResp();

If (sresp == OCP_SRESP_DVA) {

Td* rd_data_ptr = MgetSData(burst_word_length,last_chunk_of_a_burst);

}

}

// ----------------------------

// on the slave receiving side

// ----------------------------

if (SgetRequestBlocking(1)) {

Ta address = SgetMAddr();

OCPMCmdType mcmd = SgetMCmd();

If (mcmd == OCP_MCMD_RD) {

Int burst_word_length = SgetBurstLen();

// returning a read response and data

SputSResp(OCP_SRESP_DVA);

// assuming rd_data_ptr is pointed to the single-word data

SputSData(rd_data_ptr, burst_word_length, true);

SputResponseBlocking()l

}

}

This time, the MputMCmd() and MputBurstLen() calls, together, send a single-OCP-word read request to the OCP channel. When the slave receives a request, it checks whether it is a read request first. If a read request is received, the slave retrieves the word length of this read request using the SgetBurstLen() method. In this example, after the read data is obtained, the slave sends back an OCP DVA response and the single-OCP-word read data by calling the SputSResp() and SputSData()  methods, respectively.

On the master side, when a read response is arrived, the master uses the MgetSResp() and MgetSData() methods to retrieve the status of the response and the read data, respectively.

Note that for the SputSData() method and the MgetSData method, only pointer passing is allowed for now.

## Auxiliary Classes

### CommCl (tl_comm_cl.h)

This class contains the states and events used by the communication mechanism of the Channel. These states and events must not be changed by Masters and Slaves. The purpose to export these states and events is to give Masters and Slaves read access. Again, these accesses must be read only. For the intended) normal use of the Channel this class should not be changed.

### ParamCl (ocp_tl_param.h)

Transaction Level - parameter class. The parameter class is a mean for storing parameters like Master priorities or Slave addresses to other modules (e.g. the Bus). Its basic usage model is to write values to this class in the elaboration phase and read these values from the parameter class at the beginning of the simulation. The contents of this class will be refined in future releases.

The current parameters include:

CmdAccept

Purpose: 	Indicate whether full command handshake is used in TL1. If true, the slave must release the requests. If false, the requests are released by default, and a new request can be sent at every clock cycle without regard to the slave.

Default: 	True

RespAccept

Purpose: 	Indicate whether full response handshake is used in TL1. If true, the master must release the responses. If false, the responses are released by default, and a new response can be sent at every clock cycle without regard to the master.

Default: 	True

DataAccept

Purpose: 	Indicate whether full data handshake is used in TL1. If true, the slave must release the data requests. If false, the data requests are released by default, and a new data request can be sent at every clock cycle without regard to the slave.

Default: 	True

# SystemC Examples

## Complex Examples

The examples can be found in the directories ocp_tl1_examples, ocp_tl2_examples and ocp_tl2_sideband_examples. Except for the OCP_TL2_Bus, the examples focus on the usage of the communication methods of the channel, not on functionality inside the master/slave modules. The OC_TL2_Bus is an example implementation of a non-cycle-true bus.

The code can be accessed in the above-mentioned directories. The code for the generic TL channel can be found in the root directory (tl_sc). The file names in that directory start with “tl_” indicating that this is generic code, common to all layers. The user written and protocol specific files start with “ocp_tlx_”, where “x” is 1, 2, or 3 depending on the transaction layer of the example. The top level C++ files are named top_x.cpp. The C++ files for the example master and slaves have a description at the beginning, which is worth reading.

See README files in each directory for further instructions.

## Simple Examples

These examples are added to present the use of the channel at its simplest. There is one example file for each transaction layer. The example files can be found in tl_sc/simple_example directory or the release packege.

### Simple TL1

The simple TL1 example classes can be found in simple_tl1.cpp file. The example can be compiled with make file Make_tl1.gcc.

The example consists of non-pipelined master and slave connected through TL1 channel. The master requests back-to-back writes and reads, and has one idle cycle between write-read pairs. The slave can be configured either asynchronous (executes at request event) or synchronous (executes at clock). The interface configuration is simple, consisting of handshake and data signals. See the following figures for the behavior.

Figure 1. Transactions and RTL Equivalent Timing of Simple TL1 with Asynchronous SCmdAccept

Figure 2. Transactions and RTL Equivalent Timing of Simple TL1 with Synchronous SCmdAccept

### Simple TL2

The simple TL2 example classes can be found in simple_tl2.cpp file. The example can be compiled with make file Make_tl2.gcc.

The example consists of pipelined master and slave connected through TL2 channel. The master requests back-to-back write and read bursts, and has one idle cycle between write-read pairs. See the following figures for the behavior. Notice that one clock cycle corresponds to 0.1 time units in this simulation.

Figure 3. Transactions and RTL Equivalent Timing of Simple TL2

### Simple Layer-3

The simple TL3 example classes can be found in simple_tl3.cpp file. The example can be compiled with make file Make_tl3.gcc.

The example consists of a behavioral master and slave connected through TL3 channel. The master and slave go through a simple greeting message-sequence by sending blocking requests to each other. The datagram sent with each message is a C++ structure, consisting of a message string, and string length members. The simulation is naturally un-timed in TL3 level; only event sequence has meaning. This example is included for completeness, only.

The TL2 data class is actually used to create a TL3 channel; with the limitation of the current implementation that only pointer passing data transfer is allowed. Use OSCI generic TL3 channel if data copying is needed.

Figure 4. Transactions of Simple TL3

## Layer Conversions

Example converters for TL1 to TL0 and TL2 to TL1 are provided. See document sc_tl/doc/TL0_TL1_Layer_Adaptation.doc.

The current definition follows the OCP-IP 1.0 specification.

The access methods for handling OCP sideband signals are included in the provided OCP TL2 data class, but may change in the future. There is no support for OCP test signals at this time.

The current definition follows the OCP-IP 1.0 specification.